End Term Examination — Spring 2026 Semester

| Course Name: | Digital Logic Design | Course Code: | EE20203 / FLB20203 |
| --- | --- | --- | --- |
| Program: | BS (Software Engineering) | Section: | 2 |
| Semester: | 2nd Semester | Date of Examination: | 4 - June - 2026 |
| Teacher's Name: | Syed Hassaan | Total Marks: | 40 |
| Exam Duration: | 2-3 Hours | Student Name: |  |
| SAP ID: |  | Obtained Marks: |  |

INSTRUCTIONS

- Write your Name and SAP ID on all answer sheets.

- Write page number on all answer sheets.

- All questions are compulsory. Partial credit may be awarded for incomplete but relevant work.

- Show all working/steps clearly. Final answers without supporting work may not receive full marks.

- Switch off your Mobile Phone. No electronic devices allowed.

- Neat and legible work will be given preference.

Question 1:	[12 Marks]

Answer the following questions related to Boolean minimization using the Quine-McCluskey (QM) method:

(a)  Minimize the following Boolean function using the Quine-McCluskey (tabulation) method. Show all grouping tables, indicate all Prime Implicants (PIs), construct the Prime Implicant Table, identify Essential Prime Implicants (EPIs), and write the final minimized SOP expression.  [6 Marks]

F(A, B, C, D) = Σm(0, 1, 3, 7, 8, 9, 11, 15)

(b)  Apply the Quine-McCluskey method to the following function which includes don't-care conditions. Identify all Prime Implicants and give the minimized SOP expression. Don't-cares may be used to simplify but need not be covered.  [4 Marks]

F(W, X, Y, Z) = Σm(2, 6, 8, 9, 10) + Σd(0, 11, 15)

(c)  A function of 5 variables f(A, B, C, D, E) = Σm(1, 4, 6, 10, 20, 22, 24, 26) is to be minimized. Briefly explain why the Quine-McCluskey method is preferred over Karnaugh maps for this function, and list the first-column groupings (by number of 1-bits) without performing full minimization.  [2 Marks]

Question 2:	[14 Marks]

Answer the following questions on Combinational MSI Circuits — Adders, Subtractors, and Comparators:

(a)  Design a Full Adder circuit from scratch. Derive its truth table for inputs X, Y, and Cin. Write the simplified Boolean expressions for Sum (S) and Carry-out (Cout). Draw the logic circuit using only basic gates (AND, OR, XOR).  [4 Marks]

(b)  Show how a 4-bit Adder/Subtractor circuit can be built using a 4-bit ripple carry adder and XOR gates. Explain the role of the Mode select bit M: what happens when M = 0 and when M = 1. Write the operation it performs for each case.  [4 Marks]

(c)  Using two 74x283 4-bit adder ICs, design an 8-bit adder. Show how the carry-out of the lower nibble IC connects to the carry-in of the upper nibble IC. If A = 10110011 and B = 01101110, compute A + B showing the carry propagation between the two ICs.  [3 Marks]

(d)  Two 4-bit numbers A = 1011 and B = 1101 are to be compared using a 74x85 magnitude comparator. Determine the values of the three outputs (A > B, A = B, A < B). State the values you would assign to the cascading inputs for a standalone (non-cascaded) operation.  [3 Marks]

Question 3:	[14 Marks]

Answer the following questions on MSI Combinational Building Blocks — Decoders, Encoders, Multiplexers, and Demultiplexers:

(a)  Implement the following two Boolean functions simultaneously using a single 3-to-8 decoder and OR gates only. Clearly identify which decoder output lines you are ORing for each function.  [3 Marks]

F1(A, B, C) = Σm(1, 3, 5, 7)     and     F2(A, B, C) = Σm(0, 2, 4, 6)

(b)  Design a 4-to-1 Multiplexer circuit using basic logic gates. Draw its circuit diagram. Then use a 4-to-1 MUX to implement the Boolean function F(A, B, C) = Σm(0, 2, 5, 7), where A and B are the select lines and C is available as a data variable. Determine the values to be connected to data inputs I0, I1, I2, and I3.  [4 Marks]

(c)  Using an 8-to-1 multiplexer (74151A), implement the function F(A, B, C) = Σm(1, 2, 4, 6, 7). Connect A, B, C to select lines S2, S1, S0 respectively, and determine each data input Ik (0 or 1). Verify one entry from your truth table.  [4 Marks]

(d)  A 1-to-8 demultiplexer is required but only 1-to-4 DEMUX ICs are available. Show how you would cascade two 1-to-4 DEMUX units to produce a 1-to-8 DEMUX. Label all data inputs, select lines, and output lines clearly.  [3 Marks]

— END OF EXAMINATION PAPER —

Total: 40 Marks  |  Duration: 2 Hours  |  Good Luck!